# Entry: 01/07/2021

# Outline

## Goal:
To keep a hinged panel vertical (perpendicular to the ground).

The initial conditions will be:

- start from a vertical position, and remain vertical upon impulse disturbance

- start from a small angle, and return to a vertical position

## Component of the System:

Motor: it will be used to produce a torque to counter any disturbance on the hinged panel, and return the panel to a equilibrium position. A fly wheel might be used to ensure the motor can provide larger torque for a longer priod of time.

Accelerometer: it will be use to measure the angle of the hinged panel relative to the ground

PID controller: it will be used to determine the terminal voltage of the motor/set the amount of torque on the motor.

Plant: a hinged panel fix to a surface.

## To-Do list

A Simulink simulation would be carried out to test the various gain for the PID controller before parts arrive for the simulation

Analytical analysis would be carried out to deter the PID gains and the stability of the system

Build the device

# Modelling

## Plant

Calcualte the torque on the system around the hinged point

Where:

Tplant : the total troque applied to the plant around the hinge dpoint

mplant: is the total mass of the plant

Xplant.COM: is the distance from the hinged point to he CoM of the whole plant

Tm: is the torque applied by the motor onto the system

Iplant: is the angular inertia of the plant

αplant: is the angular acceleration of the plant

∑Tplant=-mplant×g×Xplant.COM×cosθ+Tm
Iplant×αPlant=-mplant×g×Xplant.COM×cosθ+Tm

Mass and CoM location can be further break down:

Where:

mplant: mass of the entire plant

mpanel: mass of the panel

mmotor: mass of the motor

mflywheel: mass of the flywheel

mplant=mpanel+mmotor+mflywheel

The CoM of the system can be calculated by the equation below:

Where:

Xplant.CoM: the CoM of the plant form the hinged point

Xmotor.CoM: the CoM of the motor form the hinged point

Xplant.CoM=mpanel×Xpanel.CoM+mmotor+mflywheel×Xmotor.Commplant

The CoM of the plant form the hinged point can be calculated by:

The hinged panel can be model as a rectangle with a circle in the middle, with a density of ρ (mass per. unit area).

Where:

ρpanel: mass per unit area will be used, kg/m^2

d: the diameter of the motor

L: the length of the panel

W: the width of the panel

Xpanel.CoM=W×L-π×d24×ρ×L2-π×d24×ρpanelmpanel

Therefore, the equation for the CoM of the plant is:

Xplant.CoM=mpanel×W×L-π×d24×ρ×L2-π×d24×ρpanelmpanel+mmotor+mflywheel×Xmotor.Commplant

## Motor

The fundamental motor dynamic equations are:

Where:

Te: Torque produce by the motor

Jeq: The rotational inertia as seen by the motor

αmotor: angular acceleration of the motor

Beq: motor damping coeffient

ωmotor: angular velocity of the motor

TL: Torque loss sue to external factors

Vt: terminal voltage supply to the motor

Ea: armature voltage induces by the motor

Ia: armature current

Ra: armature  resistance

KE: EMF constant

KT: Torque constant

Te=Jeq×αmotor+Beq×ωmotor+TL
Vt=Ea+Ia×Ra
Vt=KE×ωmotor+TeKT×Ra

Jeq in this case is the rotational inertia of the motor core plus the fly wheel, this mean Tmotor is equals to Jeq * αmotor for a direct drive fly wheel.

The motor dynamic equation would be:
Vt=KE×ωmotor+Jeq×αmotor+Beq×ωmotor+TLKT×Ra
Vt=KE×ωmotor+Tm+Beq×ωmotor+TLKT×Ra

# Entry 02/07/2021

# Choosing motor:

The budget of this project is limited, so a hobby motor would be used to keep the cost down. By browsing Jaycar’s webpage, the cheapest motor that can deliver a reasonable amount of torque is YM2707 which delivery 0.01Nm of stall torque.

Not a lot but, after some prelimary calculation, it would be able to handle about 70g of mass with Xplant.CoM of 0.08m at ±10 degrees from vertical position.

# Calculate motor parametrs:

The following constant are need for Simulink simulation: KE, KT, Ra, Jeq, Beq.

### From the data sheet of YM2707:

KT: 31.885 Grcm/A = 0.0031885 Nm/A

Max torque: 99.906Grcm = 0.01Nm

Max RPM: 12000rpm = 1257 rad/s

By measuring the armature resistance on the motor: 1.4 ohms

### Calculate KE, Beq

Based on the no load parameter of the data sheet, no load ω is 1257 rad/s, with current of 0.22A @ 4.5V

Vt=KE×ωmotor+Ia×Ra
Vt=KE×ωmotor+Jeq×αmotor+Beq×ωmotor+TLKT×Ra
4.5=KE×1257+0+Beq×1257+00.0031885×1.4 
by comparing coefficient

Ia=Beq×1257
0.22=Beq×1257
Beq=0.221257
Beq=558.05×10-9

Calculate KE

4.5=KE×1257+0.22×1.4
KE=4.5-0.22×1.31257
KE=0.003179

### Summary

KE: 0.003179 V s/rad

KT: 0.0031885 Nm/A

Ra: 1.4 ohms

Beq: 558.05 e-9 Nm s/rad

# Entry 03/07/2021

# Design the system

Goal update:

The whole system would be about 70g = 0.007kg (panel + motor + flywheel). Where the motor is about 30g, panel about 20g, and the flywheel about 20g

The system would be able to recover from ±10 degrees from vertical position

Calculate Xplant.COM and Xmotor.COM of the system

Equilibrium position at 80 deg from horizontal, and estimate the location of CoM of the system

Tm=mplant×g×Xplant.CoM×cos80
0.01=0.07×9.81×Xplant.CoM×cos80
Xplant.CoM=0.010.07×9.81×cos80
Xplant.CoM=0.0839
Xplant.CoM≈0.08 m
Base on the location of CoM of the system, calculate the required location of Xmotor.CoM

Calculate ρpanel

ρpanel=mpanelApanel
ρpanel=0.020.15×0.05-0.0122×π
ρpanel=2.837kgm2

Solve for the equation for Xmotor.CoM

Xplant.CoM=mpanel×W×L-π×d24×ρ×L2-π×d24×ρpanelmpanel+mmotor+mflywheel×Xmotor.Commplant
Xplant.CoM=0.082m

# Fusion 360 modelling

After a few iterations, this is the final design with approx. physical properties

General

Part Number	Panel

Physical

Mass	18.283 g

Volume	24088.075 mm^3

Density	0.001 g / mm^3

Area	17570.531 mm^2

World X,Y,Z	0.00 mm, 0.00 mm, 0.00 mm

Center of Mass	0.00 mm, 1.50 mm, 64.138 mm

Moment of Inertia at Center of Mass   (g mm^2)

Iyy	50127.173

Moment of Inertia at Origin   (g mm^2)

Iyy	1.253E+05

General

Part Number	Motor

Physical

Mass	48.996 g

Volume	12249.07 mm^3

Density	0.004 g / mm^3

Area	3009.646 mm^2

World X,Y,Z	0.00 mm, 0.00 mm, 0.00 mm

Center of Mass	1.252E-09 mm, 13.446 mm, 82.00 mm

Moment of Inertia at Center of Mass   (g mm^2)

Iyy	3517.848

Moment of Inertia at Origin   (g mm^2)

Iyy	3.330E+05

Physical

Mass	11.755 g

Volume	15488.023 mm^3

Density	0.001 g / mm^3

Area	12296.233 mm^2

World X,Y,Z	0.00 mm, 0.00 mm, 0.00 mm

Center of Mass	3.638E-08 mm, -8.50 mm, 82.00 mm

Moment of Inertia at Center of Mass   (g mm^2)

Iyy	41933.026

Moment of Inertia at Origin   (g mm^2)

Iyy	1.210E+05

### Summary:

| m_panel | 18.2 | g |
| --- | --- | --- |
| m_motor | 49 | g |
| m_flywheel | 11.8 | g |
| m_plant | 0.08 | kg |
| I_panel.hinge | 1.25E+05 | g mm2 |
| I_motor.hinge | 3.33E+05 | g mm2 |
| I_flywheel.hinge | 1.21E+05 | g mm2 |
| I_plant.hinge | 5.79E-04 | kg mm2 |
| I_motor.CoM | 3517.848 | g mm2 |
| I_flywheel.CoM | 41933.026 | g mm2 |
| I_motor.CoM+ I_flywheel.CoM | 4.54509E-05 | kg m2 |
| X_plant.CoM | ≈0.08 | m |

# Entry 05/07/2021

# Simulink modelling

The control lop above will be the basis used for the Simulink simulation.

The dynamic each section is based on the equation derive from earlier

## PID controller

## Voltage Saturation

The voltage supply to the motor is limited to 4.5v as it’s the rate voltage for the motor

## Motor Dynamic

The figure above shows the characteristics of the motor dynamic when applied a step input

Note: this is when there is a flywheel attached to the motor directly, so the angular acceleration curve is different without the flywheel

The angular velocity – torque characteristic curve roughly matched the spec sheet. It would be interesting to compare this to the actual graph

## Plant Dynamic

Angles are in radians, the output graph matches the behaviour of the real system a part from non-stop oscillation.

## Update: 11/07/2021

Could include a damping term to ensure the oscillation would end at some point

The oscillation eventually die out around -pi/2 which is -90degree (eg. lowest point) this match the real plant.

# Entry 07/07/2021

# Choosing parts:

## Motor driver

A full H bridge will be used to control the motor to allow forward and reverse direction od the motor. The component must be rated for 4.5V at up to 3A to ensure it can delivery the maximum current drawn by the motor.

A variant of cheap IC L298 will be used to drive the motor, more specifically https://www.aliexpress.com/item/1005001621936295.html?

It should be fine, if not we can supply 3V to the motor to ensure the IC wouldn’t be fried.

## Accelerometer

An Arduino mega will be used to control the motor driver, so a compatible Accelerometer will be used.

ADXL3xx accelerometer will be used, specifically: ADXL345

https://www.aliexpress.com/item/32452794842.html

which is a 3 axis accelerometer that utilise i2c or spi transmission protocol. This should enable the panel to measure the angle of the panel relative to the horizontal panel.

# Entry 08/07/2021

## Analytical analysis

Analysis the stability of the system, and how it reacts to various input/disturbance

## Plant

From earlier, the equation describe dynamic of the panel movement is:

Iplant×αPlant=-mplant×g×Xplant.COM×cosθplant+Tm
L{Iplant×αPlant}=L{-mplant×g×Xplant.COM×cosθplant}+L{Tm}

Given all initial conditions are 0 except angel of the plant is a value of 90 deg

Linearise cos(θ) to mθ+c around θ = 90deg = 1.57rad

cosθ≈m×θ+c
cosθ≈dcosθd θ×θ+c
cosθ≈-sin⁡(θ)×θ+x
linear rise around θ = 1.57rad
cosθ≈-1×θ+1.57

LIplant×αPlant=L-mplant×g×Xplant.COM×-θ+pi2+LTm
Iplant×s2×Θplants+s×θ0=-mplant×g×xplant.CoM-Θs+pi2×s+τm(s)

This make the equation impossible to transform to the term ϴplant(s)/τm(s).

### Redefine reference point for θplant

Require redefining the point of reference such that 0 rad is at vertical position.

Iplant×αPlant=-mplant×g×Xplant.COM×sinθplant+Tm
L{Iplant×αPlant}=L{-mplant×g×Xplant.COM×sinθ}+L{Tm}

Now all initial condition will be 0

Linearise sin(θ) around θ=0deg

sinθ≈m×θ+c
sinθ≈cosθ×θ+c
linearise around θ = 0deg

sinθ≈θ

LIplant×αPlant=L-mplant×g×Xplant.COM×θ+LTm
Iplant×s2×Θplants=-mplant×g×xplant.CoMΘs+τms
Θplantss2×Iplant+mplant×g×xplant.CoM=τms
Θplant(s)τms=1s2×Iplant+mplant×g×xplant.CoM

## Motor

From earlier, the equation for the dynamic of motor is:

Vt=KE×ωmotor+Tm+Beq×ωmotor+TLKT×Ra

Since the flywheel will be directly driven by the motor, TL term can be omitted
LVt=LKE×ωmotor+LTm+Beq×ωmotorKT×Ra
Vts=KE×Ωmotors+τmotor(s)×RaKT+Ωmotors×Beq×RaKT

Where:

Tm=Iflywheel.CoM+Imotor.CoM×αmotor

d ωmotordt=αmotor
Ld ωmotordt=LTmIflywheel.CoM+Imotor.CoM
s×Ωmotors=τmotorsIflywheel.CoM+Imotor.CoM
Ωmotors=τmotorss×(Iflywheel.CoM+Imotor.CoM)

Therefore:

Vts=KE+Beq×RaKT×τmotorss×(Iflywheel.CoM+Imotor.CoM)+τmotors×RaKT
τmotorsVts=1KE+Beq×RaKTs×(Iflywheel.CoM+Imotor.CoM)+RaKT
τmotorsVts=KT×s×(Iflywheel.CoM+Imotor.CoM)KE×KT+Beq×Ra+Ra×s×(Iflywheel.CoM+Imotor.CoM)

## Control loop

Since the reference angle will be 0, I would be interested in the repose of the system to different disturbance. (test for ability to recover)

# Entry 11/07/2021

## Transfer function

### The input is the disturbance

Ts=TPlantDynamic1+TPController×TMotorDynmic×TPlantDynamic
Ts=1s2×Iplant+mplant×g×xplant.CoM1+1s2×Iplant+mplant×g×xplant.CoM×KT×s×(Iflywheel.CoM+Imotor.CoM)KE×KT+Beq×Ra+Ra×s×(Iflywheel.CoM+Imotor.CoM)×K
Ts=(KE×KT+Beq×Ra+Ra×s×(Iflywheel.CoM+Imotor.CoM))(s2×Iplant+mplant×g×xplant.CoM)×(KE×KT+Beq×Ra+Ra×s×(Iflywheel.CoM+Imotor.CoM))+K×KT×s×(Iflywheel.CoM+Imotor.CoM)

All location of the close loop poles are expected to the on the left hand side of the imaginary axis, this mean the system will be stable.

# Entry 13/07/2021

## Root Locus of the system

From earlier, the value of the variables are:

| I_plant.hinge | 5.79E-04 | kg mm2 |
| --- | --- | --- |
| M_plant | 0.08 | kg |
| x_plant.com | 0.08 | m |
| I_motor.CoM+ I_flywheel.CoM | 4.54509E-05 | kg m2 |
| K_T | 0.0031885 | Nm/A |
| K_E | 0.003179 | V s/rad |
| B_eq | 558.05 e-9 | Nm s/rad |
| R_a | 1.4 | ohms |

### OLTF

Ts= TMotorDynmic×TPlantDynamic
Ts= KT×s×(Iflywheel.CoM+Imotor.CoM)KE×KT+Beq×Ra+Ra×s×(Iflywheel.CoM+Imotor.CoM)×1s2×Iplant+mplant×g×xplant.CoM
Ts=0.003179×4.54509×10-5×s0.003179×0.0031885+558.1×10-9×1.4+1.4×s×4.54509×10-5×s2×5.79×10-4+0.08×9.81×0.08
Ts=1.4×10-7s1.0918×10-5+6.363×10-5s×s2×5.79×10-4+0.0628
Ts=1.4×10-7s6.363×10-5 s3+1.0918×10-5s2+4×10-6s+6.86×10-7

### Root locus plot

The system would therefore be stable for all values of K, this is a good reflection of the real system.